//1. LCM code

DATA SEGMENT

NUM DW 5,4

LCM DW 2 DUP(0)

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:

MOV AX,DATA

MOV DS,AX

MOV DX,0

MOV AX,NUM

MOV BX,NUM+2

UP:

PUSH AX

PUSH DX

DIV BX

CMP DX,0

JE EXIT

POP DX

POP AX

ADD AX,NUM

JNC DOWN

INC DX

DOWN:

JMP UP

EXIT:

POP LCM+2

POP LCM

MOV AX,LCM

MOV AH,4CH

INT 21H

CODE ENDS

END START

//2.GCD code

DATA SEGMENT

NUM1 DW 000AH

NUM2 DW 0004H

GCD DW ?

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:MOV AX,DATA

MOV DS,AX

MOV AX,NUM1

MOV BX,NUM2

COMPARE:CMP AX,BX

JE EXIT

JB SWAP

DIVIDE:MOV DX,0H

DIV BX

CMP DX,0

JE EXIT

MOV AX,DX

JMP COMPARE

SWAP:XCHG AX,BX

JMP DIVIDE

EXIT:MOV GCD,BX

MOV AH,4CH

INT 21H

CODE ENDS

END START

//3.Factorial code

DATA SEGMENT

X DW 06H

FACT DW ?

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE, DS:DATA

START:

MOV AX, DATA

MOV DS, AX

MOV AX, 01H

MOV CX, X

UP:

MUL CX

LOOP UP

MOV FACT, AX

MOV AH, 4CH

INT 21H

CODE ENDS

END START

//4.Square_Cube code

DATA SEGMENT

X DW 04H

SQUARE DW ?

CUBE DW ?

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:

MOV AX,DATA

MOV DS,AX

MOV AX,X

MOV BX,X

MUL BX

MOV SQUARE,AX

MUL BX

MOV CUBE,AX

MOV AH,4CH

INT 21H

CODE ENDS

END START

//.5 positive/negative code

DATA SEGMENT

NUM DB 12H

MSG1 DB 10,13,'DATA IS POSITIVE $'

MSG2 DB 10,13,'DATA IS NEGATIVE $'

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:

MOV AX,DATA

MOV DS,AX

MOV AL,NUM

ROL AL,1

JC NEGA

MOV DX,OFFSET MSG1

JMP EXIT

NEGA:

MOV DX,OFFSET MSG2

EXIT:

MOV AH,09H

INT 21H

MOV AH,4CH

INT 21H

CODE ENDS

END START

//6.Odd/Even code

DATA SEGMENT

X DW 27H

MSG1 DB 10,13,'NUMBER IS EVEN $'

MSG2 DB 10,13,'NUMBER IS ODD $'

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:

MOV AX,DATA

MOV DS,AX

MOV AX,X

TEST AX,01H

JNZ ODD

LEA DX,MSG1

MOV AH,09H

INT 21H

JMP LAST

ODD:

LEA DX,MSG2

MOV AH,09H

INT 21H

LAST:

MOV AH,4CH

INT 21H

CODE ENDS

END START

//NORMAL ADD MID

DATA SEGMENT

NUM1 DW 1234H

NUM2 DW 5678H

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:MOV AX,DATA

MOV DS,AX

MOV CX,NUM1

MOV DX,NUM2

MOV DI,0500H

MOV AX,CX

ADD AX,DX

MOV [DI],AX

MOV AH,4CH

INT 21H

CODE ENDS

END START

//7.Equation code

DATA SEGMENT

X DB 28

Y DB ?

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:

MOV AX,DATA

MOV DS,AX

MOV AL,X

SUB AL,30

NEG AL

MOV BL,5

MUL BL

MOV BL,9

DIV BL

ADD AL,7

MOV Y,AL

EXIT:

MOV AH,4CH

INT 21H

CODE ENDS

END START

//EQUATION2

DATA SEGMENT

C DB 5

D DB 6

Y DB ?

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:

MOV AX,DATA

MOV DS,AX

MOV AL,C

MOV BL,5

MUL BL

MOV BL,D

DIV BL

MOV CL,AL

MOV AL,D

MOV BL,6

MUL BL

ADD CL,AL

MOV AL,C

MOV BL,7

MUL BL

SUB CL,AL

MOV Y,CL

MOV AH,4CH

INT 21H

CODE ENDS

END START

//AvgCalculation code

DATA SEGMENT

SUM DW 0

AVG DW 0

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:

MOV AX,DATA

MOV DS,AX

MOV CX,50

MOV BX,1

MOV AX,0

SUM_LOOP:

ADD AX,BX

INC BX

LOOP SUM_LOOP

MOV SUM,AX

MOV AX,SUM

MOV CX,50

DIV CX

MOV AVG,AX

MOV AH,4CH

INT 21H

CODE ENDS

END START

//Fibonacci Series

DATA SEGMENT

FIB DW 15 DUP(0)

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:

MOV AX,DATA

MOV DS,AX

MOV AX,0

MOV FIB[0],AX

MOV AX,1

MOV FIB[2],AX

MOV CX,13

MOV SI,4

FIB_LOOP:

MOV AX,FIB[SI-4]

MOV BX,FIB[SI-2]

ADD AX,BX

MOV FIB[SI],AX

ADD SI,2

LOOP FIB_LOOP

MOV AH,4CH

INT 21H

CODE ENDS

END START

//Reverse num

DATA SEGMENT

NUM DW 1234H

REV DW 0

QUO DW 0

REM DW 0

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START: MOV AX,DATA

MOV DS,AX

MOV AX, NUM

MOV CX, 10H

THELOOP:

DIV CX

MOV REM, DX

MOV QUO, AX

XOR DX, DX

MOV AX, REV

MUL CX

ADD AX, REM

MOV REV, AX

MOV AX, QUO

CMP AX, 0

JNE THELOOP

MOV AX, REV

MOV AH,4CH

INT 21H

CODE ENDS

END START

//Palindrome code

DATA SEGMENT

NUM DW 1221         ; Number to check

REV DW 0

QUO DW 0

REM DW 0

PAL_MSG DB 10,13,'PALINDROME$'

NOPAL_MSG DB 10,13,'NOT PALINDROME$'

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE, DS:DATA

START:

MOV AX, DATA

MOV DS, AX

MOV AX, NUM

MOV REV, 0

THELOOP:

XOR DX, DX           ; Clear DX for DIV

MOV CX, 10

DIV CX               ; AX / 10 ? quotient AX, remainder DX

MOV REM, DX

MOV QUO, AX

MOV AX, REV

MOV BX, 10

MUL BX                ; AX = REV * 10

ADD AX, REM

MOV REV, AX

MOV AX, QUO

CMP AX, 0

JNE THELOOP

; --- Compare original and reversed ---

MOV AX, NUM

CMP AX, REV

JE PRINT_PAL

PRINT_NOPAL:

LEA DX, NOPAL_MSG

MOV AH, 09H

INT 21H

JMP EXIT

PRINT_PAL:

LEA DX, PAL_MSG

MOV AH, 09H

INT 21H

EXIT:

MOV AH, 4CH          ; Proper terminate

INT 21H

CODE ENDS

END START

//LargestNumFind code

DATA SEGMENT

X DW 0010H,52H,30H,40H,50H

MAX DW ?

COUNT DW 5

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:

MOV AX,DATA

MOV DS,AX

MOV SI,OFFSET X

MOV CX,COUNT

MOV AX,[SI]

MOV BX,AX

NEXT:

ADD SI,2

LOOP_CHECK:

DEC CX

JZ DONE

MOV AX,[SI]

CMP AX,BX

JBE SKIP

MOV BX,AX

SKIP:

JMP NEXT

DONE:

MOV MAX,BX

MOV AH,4CH

INT 21H

CODE ENDS

END START

//SmallestNumFind code

DATA SEGMENT

X DW 0010H,52H,30H,40H,50H

MIN DW ?

COUNT DW 5

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:

MOV AX,DATA

MOV DS,AX

MOV SI,OFFSET X

MOV CX,COUNT

MOV AX,[SI]

MOV BX,AX

NEXT:

ADD SI,2

DEC CX

JZ DONE

MOV AX,[SI]

CMP AX,BX

JAE SKIP

MOV BX,AX

SKIP:

JMP NEXT

DONE:

MOV MIN,BX

MOV AH,4CH

INT 21H

CODE ENDS

END START

//Asecnding order(bubblesort) code

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:

MOV AX,DATA

MOV DS,AX

MOV CX,COUNT

DEC CX                 ; outer loop counter = n-1

OUTER:

MOV SI,0               ; SI = starting index

MOV DX,CX              ; inner loop counter

INNER:

MOV AX,NUM[SI]

MOV BX,NUM[SI+2]

CMP AX,BX

JBE SKIP_ASC            ; if AX <= BX, skip

XCHG AX,BX

MOV NUM[SI],AX

MOV NUM[SI+2],BX

SKIP_ASC:

ADD SI,2

DEC DX

JNZ INNER

DEC CX

JNZ OUTER

MOV AH,4CH

INT 21H

CODE ENDS

END START

//Descending order sort code

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:

MOV AX,DATA

MOV DS,AX

MOV CX,COUNT

DEC CX

OUTER_DESC:

MOV SI,0

MOV DX,CX

INNER_DESC:

MOV AX,NUM[SI]

MOV BX,NUM[SI+2]

CMP AX,BX

JAE SKIP_DESC      ; if AX >= BX, skip

XCHG AX,BX

MOV NUM[SI],AX

MOV NUM[SI+2],BX

SKIP_DESC:

ADD SI,2

DEC DX

JNZ INNER_DESC

DEC CX

JNZ OUTER_DESC

MOV AH,4CH

INT 21H

CODE ENDS

END START

8.16bitAddition code

DATA SEGMENT

NUM DW 1234H,0F234H

SUM DW 2 DUP(0)

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:

MOV AX,DATA

MOV DS,AX

MOV AX,NUM

MOV BX,0

ADD AX,NUM+2

JNC DOWN

INC BX

DOWN:

MOV SUM,AX

MOV SUM+2,BX

MOV AH,4CH

INT 21H

CODE ENDS

END START

// 32BitAddition code

DATA SEGMENT

NUM1 DW 0FFFFH,0FFFFH

NUM2 DW 1111H,1111H

SUM DW 4 DUP(0)

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:MOV AX,DATA

MOV DS,AX

MOV AX,NUM1

ADD AX,NUM2

MOV SUM,AX

MOV AX,NUM1+2

ADC AX,NUM2+2

JNC EXIT

MOV SUM+4,01H

EXIT:MOV SUM+2,AX

MOV AH,4CH

INT 21H

CODE ENDS

END START

//Sqrt code

DATA SEGMENT

X DW 25

SQRT DW 0

SQUARE DW 0

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:

MOV AX,DATA

MOV DS,AX

MOV AX,X

MOV CX,0       ; candidate for sqrt

SQRT_LOOP:

INC CX

MOV BX,CX

MUL BX

CMP AX,BX

JG SQRT_LOOP

DEC CX

MOV SQRT,CX

MOV AX,CX

MUL CX

MOV SQUARE,AX

MOV AH,4CH

INT 21H

CODE ENDS

END START

//16bitSubstraction code

DATA SEGMENT

NUM DW 4567H,2345H

DIF DW 1 DUP(0)

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:MOV AX,DATA

MOV DS,AX

CLC

LEA SI,NUM

MOV AX,[SI]

SBB AX,[SI+2]

MOV DIF,AX

MOV AH,4CH

INT 21H

CODE ENDS

END START

//.32bitSubstraction code

DATA SEGMENT

NUM1 DW 2345H,6762H

NUM2 DW 1111H,1111H

DIF DW 2 DUP(0)

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:MOV AX,DATA

MOV DS,AX

LEA SI,NUM1

LEA DI,NUM2

MOV AX,[SI]

MOV BX,[DI]

SUB AX,BX

MOV DIF,AX

INC SI

INC SI

INC DI

INC DI

MOV AX,[SI]

MOV BX,[DI]

SBB AX,BX

MOV DIF+2,AX

MOV AH,4CH

INT 21H

CODE ENDS

END START

// 8bitDivisionSignNum code

DATA SEGMENT

NUM1 DB -10

NUM2 DB 02

QUO DB 1 DUP(0)

REM DB 1 DUP(0)

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:MOV AX,DATA

MOV DS,AX

MOV AL,NUM1

CBW

IDIV NUM2

MOV QUO,AL

MOV REM,AH

MOV AH,4CH

INT 21H

CODE ENDS

END START

//8bitDivisionUnsignedNum code

DATA SEGMENT

NUM1 DB 72H

NUM2 DB 02H

QUO DB 1 DUP(0)

REM DB 1 DUP(0)

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:MOV AX,DATA

MOV DS,AX

MOV AL,NUM1

MOV AH,0H

DIV NUM2

MOV QUO,AL

MOV REM,AH

MOV AH,4CH

INT 21H

CODE ENDS

END START

//16BitDivisionUnsignedNum code

DATA SEGMENT

NUM1 DW 4567H,2345H

NUM2 DW 4111H

QUO DW 2 DUP(0)

REM DW 1 DUP(0)

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:MOV AX,DATA

MOV DS,AX

MOV AX,NUM1

MOV DX,NUM1+2

DIV NUM2

MOV QUO,AX

MOV REM,DX

MOV AH,4CH

INT 21H

CODE ENDS

END START

//1BitDivSigned code

DATA SEGMENT

NUM1 DW 4567H,2345H

NUM2 DW 4111H

QUO DW 2 DUP(0)

REM DW 1 DUP(0)

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:MOV AX,DATA

MOV DS,AX

MOV AX,NUM1

MOV DX,NUM1+2

DIV NUM2

MOV QUO,AX

MOV REM,DX

MOV AH,4CH

INT 21H

CODE ENDS

END START

//16bitDivSigned code

DATA SEGMENT

NUM1 DW 4567H,2345H

NUM2 DW 4111H

QUO DW 2 DUP(0)

REM DW 1 DUP(0)

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START: MOV AX,DATA

MOV DS,AX

MOV AX,NUM1 ; Move the lower bit of Dividend to AX

MOV DX,NUM1+2   ; Move the higher bit of Dividend to DX

CWD

IDIV NUM2        ; Perform the sign Division operation using IDIV

MOV QUO,AX    ; Store the quotient to AX

MOV REM,DX     ; Store the reminder to DX

MOV AH,4CH

INT 21H

CODE ENDS

END START

INPUT: Dividend - -44444444,

Divisor - 2222,

0UTPUT: AX – FE (quotient);

DX – FF (reminder)    ; Result is in two complement form.

//16BitSignedMultiplication code

DATA SEGMENT

NUM DW 1234H,1234H

PROD DW 2 DUP(0)

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:MOV AX,DATA

MOV DS,AX

LEA SI,NUM

MOV AX,[SI]

MOV BX,[SI+2]

MUL BX

MOV PROD,AX;    5A90

MOV PROD+2,DX;   014B

MOV AH,4CH

INT 21H

CODE ENDS

END START

//16BitUnsignedMultiplication code

DATA SEGMENT

NUM DW -2,1

PROD DW 2DUP(0)

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:MOV AX,DATA

MOV DS,AX

LEA SI,NUM

MOV AX,[SI]

MOV BX,[SI+2]

IMUL BX

MOV PROD,AX

MOV PROD+2,DX

MOV AH,4CH

INT 21H

CODE ENDS

END START

//binary to BCD conversion

DATA SEGMENT

BIN DW 01A9H

BCD DB 2 DUP(0)

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:

MOV AX,DATA

MOV DS,AX

MOV AX,BIN

MOV CL,64H

DIV CL

MOV BCD+1,AL

MOV AL,AH

MOV AH,00H

MOV CL,0AH

DIV CL

MOV CL,04

ROR AL,CL

ADD AL,AH

MOV AH,4CH

INT 21H

CODE ENDS

END START

//BCD to binary conversion

DATA SEGMENT

BCD DW 27H

BIN DW ?

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:

MOV AX,DATA

MOV DS,AX

MOV AX,BCD

AND AX,07H

MOV BX,AX

MOV AX,BCD

AND AX,0F0H

MOV CX,0AH

MUL CX

ADD AX,BX

MOV BIN,AX

MOV AH,4CH

INT 21H

CODE ENDS

END START

//STRING MATCH

DATA SEGMENT

STR1 DB 'HELLO$',0

STR2 DB 'HELLO$',0

MSG1 DB 10,13,'STRING MATCH$'

MSG2 DB 10,13,'STRING NOT MATCH$'

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START: MOV AX,DATA

MOV DS,AX

LEA SI,STR1

LEA DI,STR2

CHECK_LOOP:

MOV AL,[SI]

MOV BL,[DI]

CMP AL,BL

JNE NOTMATCH

CMP AL,'$'

JE MATCH

INC SI

INC DI

JMP CHECK_LOOP

MATCH:

LEA DX,MSG1

MOV AH,09H

INT 21H

JMP EXIT

NOTMATCH:

LEA DX,MSG2

MOV AH,09H

INT 21H

EXIT:

MOV AH,4CH

INT 21H

CODE ENDS

END START

//4) Reverse the bits of a byte

DATA SEGMENT

NUM DB 96H

REV DB ?

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE, DS:DATA

START:

MOV AX,DATA

MOV DS,AX

MOV AL,NUM

MOV BL,0

MOV CX,8

L1:

RCR AL,1

RCL BL,1

LOOP L1

MOV REV,BL

MOV AH,4CH

INT 21H

CODE ENDS

END START

// Move a block of data from one segment to another

DATA SEGMENT

SRC DB 10 DUP(5)

DATA ENDS

EXTRA SEGMENT

DST DB 10 DUP(?)

EXTRA ENDS

CODE SEGMENT

ASSUME CS:CODE, DS:DATA, ES:EXTRA

START:

MOV AX,DATA

MOV DS,AX

MOV AX,EXTRA

MOV ES,AX

LEA SI,SRC

LEA DI,DST

MOV CX,10

CLD

REP MOVSB

MOV AH,4CH

INT 21H

CODE ENDS

END START

//ASCENDING BUBBLE

DATA SEGMENT

ARR DW 1234H,0A23H,0456H,1F23H,0B12H

CNT DW 5

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE, DS:DATA

START:

MOV AX,DATA

MOV DS,AX

MOV CX,CNT

DEC CX

OUTER:

LEA SI,ARR

MOV BX,CX

INNER:

MOV AX,[SI]

CMP AX,[SI+2]

JBE NOSWAP

XCHG AX,[SI+2]

MOV [SI],AX

NOSWAP:

ADD SI,2

DEC BX

JNZ INNER

LOOP OUTER

MOV AH,4CH

INT 21H

CODE ENDS

END START

//FCH STORE 50 LOCATION

DATA SEGMENT

ARR DB 50 DUP(?)

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE, DS:DATA

START:

MOV AX,DATA

MOV DS,AX

LEA DI,ARR

MOV AL,0FCH

MOV CX,50

L1:

MOV [DI],AL

INC DI

LOOP L1

MOV AH,4CH

INT 21H

CODE ENDS

END START

//BCD TO BINARY

DATA SEGMENT

BCD DB 27H      ; Packed BCD

BIN DW ?

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE, DS:DATA

START:

MOV AX, DATA

MOV DS, AX

MOV AL, BCD         ; AL = 27H

AND AL, 0FH         ; AL = 07 (units)

MOV BL, AL          ; BL = units (7)

MOV AL, BCD

AND AL, 0F0H        ; AL = 20H

SHR AL, 4           ; AL = 02 (tens)

MOV AH, 00H

MOV CL, 0AH         ; 10 decimal

MUL CL              ; AX = tens × 10 = 20

ADD AX, BX          ; 20 + 7 = 27

MOV BIN, AX

MOV AH, 4CH

INT 21H

CODE ENDS

END START

///NUMBER FOUND SEARCH

DATA SEGMENT

NUMS DW 1234H,0A23H,0456H,1F23H,0B12H

SEARCH_NUM DW 0456H        ; Number to search

FOUND DB 10,13,'NUMBER FOUND$'

NOTFOUND DB 10,13,'NUMBER NOT FOUND$'

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE, DS:DATA

START:

MOV AX, DATA

MOV DS, AX             ; Initialize DS to point to DATA segment

LEA SI, NUMS            ; SI -> start of array

MOV CX, 5               ; Number of elements in array

MOV DX, SEARCH_NUM      ; DX = number to search

S_LOOP:

CMP [SI], DX            ; Compare current element with search number

JE NUMBER_FOUND         ; If equal, jump to NUMBER_FOUND

ADD SI, 2               ; Move to next word

LOOP S_LOOP             ; Repeat for all elements

; If number not found

LEA DX, NOTFOUND

MOV AH, 09H

INT 21H

JMP END_PROGRAM

NUMBER_FOUND:

LEA DX, FOUND

MOV AH, 09H

INT 21H

END_PROGRAM:

MOV AH, 4CH

INT 21H                ; Terminate program

CODE ENDS

END START

///CHARACTER FOUND

DATA SEGMENT

MSG DB 'HELLO'

CNT EQU $-MSG

SRC EQU 'E'

MSG1 DB 10,13,'CHARACTER FOUND$'

MSG2 DB 10,13,'CHARACTER NOT FOUND$'

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA,ES:DATA

START: MOV AX,DATA

MOV DS,AX

MOV ES,AX

LEA SI,MSG

MOV AL,SRC

MOV CL,CNT

MOV CH,00H

CLD

UP: SCASB

JZ DOWN

LOOP UP

LEA DX,MSG2

MOV AH,09H

INT 21H

JMP EXIT

DOWN: LEA DX,MSG1

MOV AH,09H

INT 21H

EXIT: MOV AH, 4CH

INT 21H

CODE ENDS END START

///FIBONACCI

DATA SEGMENT

FIB DW 15 DUP(0)

MSG DB 10,13,'Fibonacci Series: $'

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE, DS:DATA

START:

MOV AX, DATA

MOV DS, AX

MOV AX,0

MOV FIB[0],AX

MOV AX,1

MOV FIB[2],AX

MOV CX,13

MOV SI,4

FIB_LOOP:

MOV AX,FIB[SI-4]

MOV BX,FIB[SI-2]

ADD AX,BX

MOV FIB[SI],AX

ADD SI,2

LOOP FIB_LOOP

LEA DX, MSG

MOV AH,09H

INT 21H

MOV CX,15      ; total numbers

MOV SI,0

PRINT_LOOP:

MOV AX, FIB[SI]

CALL PRINT_NUM

MOV DL, ' '

MOV AH,02H

INT 21H

ADD SI,2

LOOP PRINT_LOOP

MOV AH,4CH

INT 21H

; -------------------------

PRINT_NUM PROC

PUSH AX

PUSH BX

PUSH CX

PUSH DX

MOV BX,10

XOR CX,CX

DIV_LOOP:

XOR DX,DX

DIV BX

PUSH DX

INC CX

CMP AX,0

JNE DIV_LOOP

PRINT_DIGIT:

POP DX

ADD DL,'0'

MOV AH,02H

INT 21H

LOOP PRINT_DIGIT

POP DX

POP CX

POP BX

POP AX

RET

PRINT_NUM ENDP

CODE ENDS

END START

//SEMESTER 1

DATA SEGMENT

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:MOV AX,DATA

MOV DS,AX

MOV BX,0B35AH

MOV CX,16

XOR AX,AX

PUSH BX

COUNT_LOOP:

ROL BX,1

JNC SKIP

INC AX

SKIP:

LOOP COUNT_LOOP

POP BX

MOV AH,4CH

INT 21H

CODE ENDS

END START

//FIND LARGE

DATA SEGMENT

NUMS DW 1234H,0A23H,0456H,1F23H,0B12H

LARGE DW ?

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:MOV AX,DATA

MOV DS,AX

LEA SI,NUMS

MOV AX,[SI]

MOV CX,4

NEXT: ADD SI,2

CMP AX,[SI]

JAE SKIP

MOV AX,[SI]

SKIP:

LOOP NEXT

MOV LARGE,AX

MOV AH,4CH

INT 21H

CODE ENDS

END START

//9 NUMBER SEMESTER

DATA SEGMENT

AB DW 100      ; AB = 100 cm

AC DW 120      ; AC = 120 cm

BD DW 80       ; BD = 80 cm

CE DW ?        ; Result CE

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE, DS:DATA

START:

MOV AX, DATA

MOV DS, AX

; Step 1: Calculate AB^2

MOV AX, AB

MOV BX, AX

MUL BX          ; AX*BX = AB^2 → DX:AX, but small number, DX=0

; Step 2: Calculate BD^2

MOV CX, BD

MOV DX, CX

MUL DX          ; DX*DX = BD^2 → DX:AX

; Since numbers small, result fits in AX, ignore high word

; Step 3: Subtract BD^2 from AB^2

MOV BX, AX      ; BX = BD^2

MOV AX, AB

MUL AB          ; AX = AB^2 again

SUB AX, BX      ; AX = AB^2 - BD^2 = 3600

; Step 4: Take square root manually (AX=3600)

; Since 3600 is perfect square, sqrt(3600)=60

MOV AX, 60      ; AD = 60

; Step 5: Calculate CE = AC - AD

MOV BX, AC

SUB BX, AX      ; BX = AC - AD = CE

MOV CE, BX      ; Store result in memory

; Exit program

MOV AH, 4CH

INT 21H

CODE ENDS

END START

//ASENDING 6.DATA SEGMENT

x DW 42H,34H,26H,17H,09H

LEN EQU 05

ASCD DB 10 DUP(0)

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:

MOV AX,DATA

MOV DS,AX

MOV BX,LEN-1

MOV CX,BX

UP1:

MOV BX,CX

LEA SI,X

UP:

MOV AX,[SI]

MOV DX,[SI+2]

CMP AX,DX

JB DOWN

MOV [SI],DX

MOV [SI+2],AX

DOWN:

INC SI

INC SI

DEC BX

JNZ UP

DEC CX

JNZ UP1

MOV AH,4CH

INT 21H

CODE ENDS

END START

// STRING PALINDROME

DATA SEGMENT

STR1 DB 'MADAM'          ; Amader input string

LEN DW 5                 ; String er length

MSG1 DB 'IT IS PALINDROME$'

MSG2 DB 'NOT PALINDROME$'

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE, DS:DATA

START:

MOV AX, DATA

MOV DS, AX

MOV ES, AX               ; Extra Segment e Data Segment initialize kora

LEA SI, STR1             ; SI point korbe string er shurute

LEA DI, STR1

ADD DI, LEN

DEC DI                   ; DI point korbe string er sheshe

MOV CX, LEN

SHR CX, 1                ; Length ke 2 diye bhag kora (shudhu ordhek check korlei hoy)

CHECK:

MOV AL, [SI]             ; Shurur diker character nilam

MOV BL, [DI]             ; Shesh diker character nilam

CMP AL, BL               ; Compare kora

JNE NOT_PALINDROME       ; Jodi na ???? tobe not palindrome

INC SI                   ; Shuru theke samne agano

DEC DI                   ; Shesh theke pichone asha

LOOP CHECK

; Jodi loop sesh hoy tar mane eta palindrome

LEA DX, MSG1

MOV AH, 09H

INT 21H

JMP EXIT

NOT_PALINDROME:

LEA DX, MSG2

MOV AH, 09H

INT 21H

EXIT:

MOV AH, 4CH

INT 21H

CODE ENDS

END START

//ascending

DATA SEGMENT

ARR DW 42H,34H,26H,17H,09H   ; Unsorted array

LEN DW 5                    ; Number of elements

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE, DS:DATA

START:

MOV AX, DATA

MOV DS, AX

MOV CX, LEN

DEC CX                      ; Outer loop count = LEN-1

OUTER:

MOV SI, 0

MOV BX, CX                  ; Inner loop counter

INNER:

MOV AX, ARR[SI]

MOV DX, ARR[SI+2]

CMP AX, DX

JBE NO_SWAP                 ; Ascending: AX ≤ DX → no swap

; Swap

MOV ARR[SI], DX

MOV ARR[SI+2], AX

NO_SWAP:

ADD SI, 2

DEC BX

JNZ INNER

DEC CX

JNZ OUTER

MOV AH, 4CH                 ; Exit program

INT 21H

CODE ENDS

END START